G3APP software (Android version) instructions

Overview

G3APP software is developed for our outdoor 3G wireless hunting camera. Users can set or change parameters through remote control via G3APP.

Software installation

Users can get software: G3APPXXXXXXXX.apk from our website or dealers. XXXXXXXX on behalf of the date is to identify the version number, such as 20170828 means version of August 28, 2017. After getting the software, then copy the software into the phone and run the APK to install it.

Main interface and camera add

After the user installs the software, click on the software to enter the main interface, as shown below. Click the "+" sign in the top right corner to add the camera you need to monitor. In the Add Camera interface, set the name of the camera, the SIM card number in the camera, the user’s receiving number and the camera's IMEI. After setting up, click OK to confirm then you can add a camera in the main interface. Such as camera1.

Parameter settings

After setting up the camera, click on the camera name, such as camera1 to set the camera parameters, as shown below. In this interface you can choose to do the camera basic parameter settings, network settings, camera name changes and delete the camera.

Basic parameter settings

Click Camera Setting to do parameters setting.

- Mode

Set work mode for camera, there are three work modes for selection：Camera、Video、Camera+Video.

- Image Size

Photo resolution for choosing: 3MP, 5MP, 8MP, 12MP, 16MP.

- Video Size

Video size for choosing: 1080P, 720P, WVGA, VGA.

- Shooting No

The number of continuous shooting can be 1 to 6 sheets. (It will be only 1 to 3 sheets when choose resolution as 16MP)

- Video Length

- Video length for choosing: 5s, 10s, 15s, 20s, 30s, 45s, 1min, 2min, 3min, 5min, 10min.

- Time Lapse

- Interval shooting can be selected off, the opening interval from 5s,10s,15s,30s, 1min,2min,5min,10min,20min,30min, 1h,2h,6h,12h,18h,24h optional.

- PIR Interval

- PIR sensing interval can also be closed, the opening time interval from 3s,5s,10s,15s,30s,1min,2min,5min,10min,20min,30min,1hour optional.

- Note: Time lapse and PIR interval can’t be turned on at the same time, but can be turned off at the same time.

- Device No.

- Device No. option can set the name of the camera, it should be noted that the camera name can only be any of the four characters from 0 ~ 9, A ~ Z.

- Timer

The camera supports two sets of timers: Timer1 and Timer2, when turned on, you can set the timing on and end time options, users need to set their regular work interval. The time code form will show different styles for different phones.

Note: When the timer is turned on, the camera does not work outside of the timer, and users also need to ensure the time of the camera to match the local time.

- Auto Power Off

In TEST mode, when the keyboard without any operation in a period of time, the camera will automatically shut down to save power, users can set according to need.

- PIR Level

Three levels of PIR sensitivity: low, normal and high.

- Password Set

Set the camera's password, only from 0 to 9, A ~ Z (capital letters) any of the four characters.

- TV Mode

TV output system with PAL and NTSC standard selection.

- 4.2 Network parameter settings

Select Net Setting to set the camera's network settings. Network parameters are divided into Email, MMS and SMS three application types.

Email

Select each item, click to enter the input interface, fill in the information one by one.

NET APN should be filled in according to local operators. For example: China Unicom fill in UNINET, AT&T fill wap.cingular.

Send Server and Port are the mailbox server and the sending port for sending the mailbox. Please consult your local operator. For example: QQ mailbox fill smtp.qq.com, Port 465; GMAIL mailbox for smtp.gmail.com, Port 465

Send Mailbox and Send Password fill in the appropriate mailbox and password.

Receive Mailbox and CC Mailbox is receiving and cc mailbox.

Picture Type is the resolution of sending photo, Original is the original photo taken by camera, VGA is thumbnail, sending VGA can save traffic.

SMS, when using Email to send pictures, you can also send a short message to the user specified mobile phone, notify the user that an email to the specified mailbox, you can choose to close.

Send Interval: Interval of sending pictures to the mailbox. Select Instant to send immediately, camera will start the 3G network function directly and send photos. The timing of the time is from last shoot, camera then begin to send photos if no shoot in the specified period of time. Then camera may take more than one photo before this period of time, each time you open the 3G network, you can send three emails, each email can contain 10 original photos. Interval transmission can reduce the use of 3G network, to achieve the purpose of saving.

APN Account and APN Password, fill in the APN account and password according to the local operator. For example, China Unicom is empty, no need to fill; AT & T's APN account for wap @ cingulargprs.com, APN password for CINGULAR1

MMS

When select MMS mode, camera will send the image to the receiving phone number. Specific information is subject to local operators.

| MMS parameter | China Unicom | AT&T |
| --- | --- | --- |
| MMS APN | UNINET |  |
| MMS Url |  |  |
| MMS Proxy |  |  |
| MMS Port |  |  |
| APN Account |  |  |

SMS

SMS mode, camera will remind user that it took new image, SMS does not contain image information.

Send Interval: please choose instant.

Renamed the camera

The well-setting camera can be renamed. Click the Rename option under the Setting List interface. User can change the camera name, the number of the SIM card in the camera, the number of sending configuration file, and the IMEI information. Input box for the initial set of information, user can fill in new information after deleting the information in input box.

Delete the camera

You can delete the unwanted camera, click Delete Camera to select OK to delete.

Parameters’ sending and receiving

There is a three-point icon at the top right of each parameter setting page, click the icon, user can send, save or save as default the setting parameters.

Send: Use the phone's SMS function to send parameters to the camera.

Save: Save the parameters of current page settings.

Save Default: Save current page settings as the default value, which will be used directly for the next camera set until be changed.

Parameter receives

When Camera in working mode, it will automatically change the parameters. And camera will use the new parameters in next shot.

In test mode, users can make the camera get parameters configuration via G3APP.

Select “Get Cfg” option on Set page from menu, after you click “OK” it will pop up: "Do you want to get configuration”?

Then select OK, the camera will turn off display screen to wait parameters configuration information. In the process of acquiring, the blue Led will continuously flash, it means that camera is getting the parameters. After information completed and the camera will automatically power on, changed network parameters information will be displayed on the camera, the basic parameters can be confirmed by viewing.

Wait for more than 2 minutes without information receiving, the camera will automatically turn on also.

Software version

In camera interface, click the bottom right corner of “My” region, you can query the current software version.

The Current version is “G3 Android v1.0.0”

Attention

All information sending effected by operator, signal strength and other factors, may not be able to receive in time.